Experiment 3

Designing a Quarter Wave Transformer by Varying Frequency, Input Impedance and Characteristic Impedance

Test Case 1

Design a quarter-wave transformer for an input impedance (Zin) of 50 Ω and a load impedance (ZL) of 100 Ω. Analyze the transformer's performance as the operating frequency is varied across {100 MHz, 200 MHz, 300 MHz, 500 MHz}.

Code -

clear; clc;

Zr=100; Zs=50;

Ro=sqrt(Zs*Zr);

f=[100 200 300 500]*10^6;

for i=1:4

lambda=300/(f(i)*10^-6);      // wavelength in metres

L=lambda/4;                   // quarter-wave length

printf('f=%d MHz  Ro=%f ohm  L(lambda/4)=%f m\n',f(i)*10^-6,Ro,L);

end

Output:

ZR = 100 Ω, Zin = 50 Ω  →  Ro = 70.71 Ω (constant, independent of frequency)

| Frequency (MHz) | Wavelength λ - Sim (m) | Length of λ/4 transformer - Theory (m) |
| --- | --- | --- |
| 100 | 3.00 | 0.75 |
| 200 | 1.50 | 0.375 |
| 300 | 1.00 | 0.25 |
| 500 | 0.60 | 0.15 |

Test Case 2

For a fixed operating frequency of 2 GHz and a load impedance (ZL) of 100 Ω, design a quarter-wave transformer and evaluate its performance as the input impedance (Zin) varies through {25, 50, 75, 100} Ω. Validate that Ro = √(Zin·ZL) changes with Zin.

Code -

clear; clc;

ZL=100; f=2*10^9;

Zin=[25 50 75 100];

for i=1:4

Ro=sqrt(Zin(i)*ZL);

printf('Zin=%d ohm  Ro=%f ohm\n',Zin(i),Ro);

end

Output:

ZL = 100 Ω, f = 2 GHz  →  Variable: Zin

| Zin (Ω) | Ro (ohms) - Simulation | Ro (ohms) - Theoretical |
| --- | --- | --- |
| 25 | 50.00 | 50.00 |
| 50 | 70.71 | 70.71 |
| 75 | 86.60 | 86.60 |
| 100 | 100.00 | 100.00 |