Experiment 4

Strategize Impedance Matching with a Short-Circuited Stub through Measurement of Location and Length of the Stub

Test Case 1

Optimize impedance matching using single stub matching by finding the location and length of the stub for load impedance ZR varied from 100∠30° Ω to 350∠30° Ω, keeping the characteristic impedance Zo = 60 Ω constant (f = 100 MHz).

Code -

clear; clc;

Zo=60; f=100*10^6;

ZR=[100 150 200 250 300 350];

lo=300/(f*10^-6);   // wavelength

for i=1:6

Ls=(lo/(2*%pi))*atan(sqrt(ZR(i)/Zo));

Lt=(lo/(2*%pi))*(%pi+atan(sqrt(ZR(i)*Zo)/(ZR(i)-Zo)));

printf('ZR=%d  Ls=%f cm  Lt=%f cm\n',ZR(i),Ls*100,Lt*100);

end

Output:

Zo = 60 Ω, f = 100 MHz, λ = 3 m  →  Variable: ZR

| ZR (Ω) | Point of attachment Ls (cm) - Sim | Stub length Lt (cm) - Sim | Ls (cm) - Theory | Lt (cm) - Theory |
| --- | --- | --- | --- | --- |
| 100 | 43.53 | 202.24 | 43.53 | 202.24 |
| 150 | 48.07 | 188.76 | 48.07 | 188.76 |
| 200 | 51.07 | 181.70 | 51.07 | 181.70 |
| 250 | 53.25 | 177.34 | 53.25 | 177.34 |
| 300 | 54.92 | 174.34 | 54.92 | 174.34 |
| 350 | 56.26 | 172.13 | 56.26 | 172.13 |

Result:

Impedance matching using single stub matching was optimized by computing the stub location and length for varying load impedance ZR, keeping Zo = 60 Ω constant. Both the point of attachment and the stub length increase with ZR, though the rate of increase reduces at higher ZR.

Test Case 2

Optimize impedance matching using single stub matching by finding the location and length of the stub for characteristic impedance Zo ranging from 600 Ω to 1800 Ω, keeping the load impedance ZR = 100∠30° Ω constant (f = 100 MHz).

Code -

clear; clc;

ZR=100; f=100*10^6;

Zo=[600 900 1200 1500 1800];

lo=300/(f*10^-6);

for i=1:5

Ls=(lo/(2*%pi))*atan(sqrt(ZR/Zo(i)));

Lt=(lo/(2*%pi))*(%pi+atan(sqrt(ZR*Zo(i))/(ZR-Zo(i))));

printf('Zo=%d  Ls=%f cm  Lt=%f cm\n',Zo(i),Ls*100,Lt*100);

end

Output:

ZR = 100 Ω, f = 100 MHz, λ = 3 m  →  Variable: Zo

| Zo (Ω) | Point of attachment Ls (cm) - Sim | Stub length Lt (cm) - Sim | Ls (cm) - Theory | Lt (cm) - Theory |
| --- | --- | --- | --- | --- |
| 600 | 18.51 | 128.25 | 18.51 | 128.25 |
| 900 | 15.36 | 132.87 | 15.36 | 132.87 |
| 1200 | 13.42 | 135.43 | 13.42 | 135.43 |
| 1500 | 12.06 | 137.11 | 12.06 | 137.11 |
| 1800 | 11.05 | 138.32 | 11.05 | 138.32 |

Result:

Impedance matching using single stub matching was optimized by computing the stub location and length for varying characteristic impedance Zo, keeping ZR = 100 Ω constant. As Zo increases, the required stub location moves closer to the load while the stub length increases.